M04

GO2cam V6.10

Tutorial

M04 – Fixing plate

- Part Layout

# Quantity : 150

# Material : Au4G

- Process for Machining

Targets :

- Import 3D parts

- Application des cycles of Facing, Pointing, Drill Hole, Rework, Slotting and Contour

- Get the surface to be machined

- Basic parameters of operation

- Tool selection

| ## Import 3D parts : The import function consists of three steps: - Positioning, Creation of stock and Creation of origin. Do not quit this function until all the 3 steps are completed. Otherwise you cannot go back to completed the missing step. - Left click on File - Left click on automatic solid import Select the file ‘M04_Plaque de fixation’, if it is not visible check "File Type" |  |
| --- | --- |

| - Face support This step allows the support surface to be positioned on the mounting system to orient the Z axis. - By default, select one side of the stock. If this is not the desired side, you can click the other side of the stock. - Icon allows to get a face on the part - In this tutorial, we keep the bottom surface as the support surface - Left click on to validate the first step - Stock size and direction - Enter the value 57 in the height box - Switch the position to the top offset and type a value of 2 - The direction allows the workpiece to rotate around the Z axis. In this example, we keep the value 0. - Also set the overflow to 0. - Left click on to validate this second step - Creation of origin - Left click on to enter manual origin - Press Ctrl+Shift and select the central island arc - Enter a value of 0 in the Z value box - Left click on to validate this third step |  |
| --- | --- |

Ope 10 Facing

| ## Start : - Left click on the sub-menu Standard |  |
| --- | --- |
| ## Geometry selection : - Left click on - Select the upper surface of the part |  |
| ## Tool selection: - Left click on - Left click on Face Mill Cutter - Automatic creation of Ø 63 tool |  |
| ## Machining cycle selection: - Left click on - Left click on Facing Pocket |  |
| ## Toolpath calculation: - Left click on Cycle Calculation |  |

Ope 20    External profile

| ## Geometry selection : The mode is still active. - Left click on - In the drop-down menu of profile creation mode, select Bottom Face - Left click on the plane outside It displays the outer profile of the island and create an overflow at all edges of the part Height is automatically acquired |  |
| --- | --- |
| ## Tool selection : - Left click on - Left click on Flat End Mill - Double-Click on Diameter and type in 20 |  |
| ## Machining cycle selection : - Left click on - Left click on Pocket. - In the strategy setting, change the allowance value in Z to 0 |  |
| ## Toolpath calculation : - Left click on Cycle Calculation |  |

Ope 30 Inside Pocket

| ## Geometry selection : The mode is still active. - Left click on - In the drop-down menu of profile creation mode, select bottom face - Left click on the flat inside pocket It displays internal profile of pocket and external profile of island Height is automatically acquired |  |
| --- | --- |
| ## Tool selection : - Left click on - Left click on Flat End Mill - Left click twice in Diameter then type 20 |  |
| ## Machining cycle selection : - Left click on |  |
| ## Toolpath calculation : - Left click on Cycle Calculation |  |

Ope 40 Inside Pocket rework

| ## Geometry selection : The mode is still active. - Left click on - In the machining tree, left click on the arrow before the cycle name - Left click on and drag the pocket/island icon of the last pocket operation to the window on right side. The profile and height of pocket operation will be copied into this new operation. |  |
| --- | --- |
| ## Tool selection : - Left click on - Left click on Flat End Mill |  |
| ## Machining cycle selection : - Left click on - Set the Z Step (Ap) to 0. - In the strategy setting, change the reference diameter to 20. |  |
| ## Toolpath calculation : - Left click on Cycle Calculation |  |

Ope 50    Bottom Finishing of inside pocket

| ## Geometry selection : The mode is still active. - Left click on - In the machining tree, left click on thearrow before the cycle name. - Left click on and drag the pocket/island icon of the last pocket rework operation to the window on right side. The profile and height of the pocket rework operation will be copied into the new operation. |  |
| --- | --- |
| ## Tool selection : - Click and drag the Pocket Rework tool to enter the big window on right side. The tool from the last Pocket Rework operation is now copied in this new operation |  |
| ## Machining cycle selection : - Left click on - Left click on Pocket. - Set the Z stock allowance to 0. - In the strategy setting, change the Stepover (Tool ratio) to 0.6. - Set the Calculation of Z Steps to «Residual material » and thickness to 0.5 mm. - Set the Island reworking and Bottom reworking to « Yes ». |  |
| ## Toolpath calculation : - Left click on Cycle Calculation |  |

Ope 60 Inside Contouring

| ## Geometry selection : - Left click on sub menu - Left click on - Left click on the arrow before the cycle name - Left click and drag the Pocket/Island icon from the Pocket Rework operation to the big window on the right side. |  |
| --- | --- |
| ## Tool selection : - Click and drag the Rework Pocket tool to enter the big window on right side. The tool from the last Pocket Rework operation is now copied in this new operation. |  |
| ## Machining cycle selection : - Left click on - Left click on Contouring. - In the strategy, set the Z Step to 0. |  |
| ## Toolpath calculation : - Left click on Cycle Calculation |  |

Ope 70 Outside Contouring

| ## Geometry selection : The is still active. - Left click on - In the drop-down menu of profile creation mode, select Lateral Face - Left click on an outside vertical surface, if necessary, click the green arrow to reverse the direction - Repeat this step on the arrival surface (red arrow on the image) - Click to change the entry/exit point of the tool on the profile to same as on the image. - Left click on - Click and select a face or point with a height of - 30 Profile is created as the external profile of the island and create an overflow at all edges of the room |  |
| --- | --- |
| ## Tool selection : - Left mouse click on - Left mouse click on Flat End Mill Select the Standrd tool 6.5 mm. |  |
| ## Machining cycle selection: - Left click on - Left click on Contouring. Inside the strategy, in the movement tab set the parameters as shown on the image. |  |

| ## Toolpath calculation : - Left click on Cycle Calculation |  |
| --- | --- |

Ope 80 Drilling 2 holes

| ## Geometry selection : - Left click on the mode and select Manual Holes - Left click on - Left click on to activate multiple selection Left click on any of the 3 holes, select all holes of the same diameter |  |
| --- | --- |
| ## Tool selection : - Left click on - Left click on Drill - Left click on to cancel the filter and select a drill tool with diameter of 5 |  |
| ## Machining cycle selection : - Left click on - Left click on Drilling - On the strategy page, select "Diameter" in depth calculation, and then type – 3 mm in Retract |  |
| ## Toolpath calculation : - Left click on Cycle Start |  |

Ope 90    Tapping

| ## Geometry selection : - Manual Holes is still activated. - In the machining tree, left click the arrow in front of the cycle name. - Left click and drag the Hole selection of the drilling operation until you enter the window on right side. - The 3 holes are selected. |  |
| --- | --- |
| ## Tool selection : - Left click on - Left click on Tap |  |
| ## Machining cycle selection : - Left click on - Left click on Tapping - Set the depth to 0 - On the strategy page, select "Diameter" in depth calculation, and then type - 3 in retract. |  |
| ## Toolpath calculation : - Left click on Cycle Calculation |  |

Ope 100    Chamfering

| ## Geometry selection : - Left click on sub menu - Left click on - In the drop-down menu of profile creation mode, select Sharp edges - Left click on Select by click - Left click on both sides as shown in the figure. - If the direction of the profile is opposite Left click on - Left click in background and Left click on |  |
| --- | --- |
| ## Tool selection : - Left click on - Left click on Chamfering Mill In the tool settings, switch the Piloted Diameter on Tip to "Yes" |  |
| ## Machining cycle selection : - Left click on - Left click on Chamfering - Edit the strategy and change the Depth Recalculation to the Z Delta, the delta value is 1, Enter a value of 0 in the width of the chamfer |  |
| ## Toolpath calculation : - Left click on Cycle Calculation |  |

Ope 110    Slotting

| ## Geometry selection : - Left click on the sub menu - Left click on - In the drop-down menu of profile creation mode, select the edges path - Left click on the bottom edge of the internal groove. - Left click on - Left click on top Z then click the edge at the top of the groove to obtain - 12.5 - Left click on bottom Z then click the edge at the bottom of the groove to obtain - 17.5 |  |
| --- | --- |
| ## Tool selection : - Left click on - Left click on Side and Face Mill Cutter |  |
| ## Modification of tool: - In Tool selection, left click twice on the tool icon of the Side and Face Mill Cutter - Modify the different parameters of the tool, as shown on the image - Click on the icon to insert a toolholder. - Modify the different characteristics of the tool holder, as shown on the image |  |

| ## Machining cycle selection : - Left click on - Left click on Slotting. - In the strategy, change offset type to the left - Change the Slot width to 2 and the profile selected to "side" - Change Z machining to Middle+Depth+Alt - Change Z Steps to 2 - Change the lead in and lead out length to 12 in movement page |  |
| --- | --- |
| ## Toolpath calculation : - Left click on Cycle Calculation |  |

- Import of fixture

Objective:

- Guide in fixture

- Positioning of fixture

- Collision management with fixture

| ## 1.Import of fixture : - Right click on stock - Left click on vice and clamp - On the user, select «Vice_2_jaws_7_64_03266 » - Left click on |  |
| --- | --- |
| ## 2.Positioning of fixture: When importing the fixture, a window will appear - Left click on position 1 and switch to position 2 - Click on to validate |  |
| ## 3.Define the fixture movement : - Left click on to rotate the fixture along Z, or enter a value in the Rotate Z box (if necessary) - Use the axis system of the vice to perform translation on each axis (x, y, or z), or enter values in the window - Left click on each green arrow to position each jaw element relative to the workpiece or stock - Click on to validate Note : Your fixture is imported and positioned, and collision of fixture is managed |  |

- Creation of Opelist

Objective:

- Automate machining cycles

| ## Selection d’un cycle : - Right click on cycle Facing Pocket - Left click on Opelists # Left click on Export |  |
| --- | --- |
| ## Edit Opelist : # You are now in the Opelist editor. The backgroud is orange, different from the standard machining tree background. # If you click Validate, the operator will be created # Don't confirm now, but please read the following steps. |  |
| ## Include automatic reference : # We will make some changes and customizations to opelist. # Right click on Operation.OPL # Left click on Automatical Reference |  |
| - Change the Pocket/Island parameter to Auto Facing - Then click on Note : This parameter allows automatic search of solid surfaces for Face Milling. |  |
| ## Save Opelist: - Click on - Create « Aluminum » folder in Opelist folder using icon - Save your opelist in the name of Facing in « Aluminum » folder |  |
| ## Opelist of pocket cycle - According to the same principle as the Facing Pocket, create an Opelist for the second Pocket cycle (Pocket_1) and a second Opelist for the outside Contouring cycle (Contouring_1). - For automatic reference, select all the features (available only for Pocket Cycle). - Name the first Opelist ‘Pocket’ and the second Opelist ‘Contouring G41’. |  |

Simulation and NC output

| ## Simulation : - Enter the menu - Left click on - Left click or to start simulation on all machining operations - If you want to switch to step-by-step mode, use the space bar - Left click on Or press the Escape key to stop the simulation |  |
| --- | --- |

## Generation of NC Code :

- Left click on

- Select the post processor M67-FANUC from the list

- Left click on open

- Left click on Confirm

- The ISO program is generated