T02

T02

GO2cam V6.10

Tutorial

T02 – Cover

## T02 - Cover

Introduction

Welcome to this cover creation tutorial.

This tutorial has the objective to help the user:

- Learn how to use the turning environment in GO2cam

- Understand the uses and capabilities of GO2cam in turning.

- Get used to the user interface and the mouse actions by practicing on a simple example.

Extra files

In the Basic Training Pack, you can find:

- the pdf file of this tutorial

- the layout design

The tutorials are additional resources to GO2cam's online help. You can access the online help by selecting the Help menu in GO2cam or by pressing F1.

# Process for the Design:

| - Choice of the Turning environment in the homepage: - Left-click on the Turning icon the most on the right Note: the icons represent the type of product. If your licence do not include it, the product icon is greyed. The software opens in design mode by default. - Left-click on Shaft - Left-click on Shaft Creation |  |
| --- | --- |
| - Sketch of the part: - Left-click on the Shaft Creation - Choice of the starting point: Z=0, D=200 - Type in 0 for Z and 200 for the Diameter - Left-click on to confirm the starting point Note: GO2cam proposes you alternate length and diameter. This choice is shown by an orange line linked to the motion of the cursor. |  |
| - Length L=22: - Left-click on , then choose horizontal segment - You can enter positive or negative values of length depending on the direction of the + sign defined by or . - The button should be in that position , if it is not, click on the button to change the direction. - Type in 22 and press Enter. |  |
| ### Diameter Ø=190: - Type in 190 and press Enter. |  |
| ### Length L=10 - Type in 10 and press Enter. |  |
| ### Groove end: 2 solutions : Enter the value with the keyboard : type in 200 and press Enter Or - Left-click on the first horizontal segment which was first created and which has the same value. |  |
| ### Length L=8: - Type in 8 and press Enter. |  |
| ### Creation of a cone L=60, Ø=270: - Left-click on - Choose the mode of creation of the cone clicking on until it changes so that you can define the cone by its length and diameter - Type in 60 for the length, 270 for the diameter and press Enter - Left-click on |  |
| ### Diameter Ø=315: - Left-click on - Type in 315 and press Enter. - You are no longer in the cone creation mode. GO2cam proposes you once again to define the length and diameter alternatively. |  |
| ### Length L=10: - Type in 10 and press Enter. |  |
| ### Diameter Ø=80: - Type in 80 and press Enter. |  |
| ### Length L=50: ### The direction of the + sign is still , there are 2 possible solutions: - Type in - 50, Enter key Or - Change the direction with a click on and Left-click on sur , which becomes - Type in 50, Enter key |  |
| ### Diameter Ø=170: We are going to make a mistake on purpose: - Type in 150 and press Enter. |  |
| ### Finishing of the sketch: To finish the part, we can click on existing elements: - Left-click on the vertical axis - Left-click on the first horizontal segment which was first created - Left-click on - Esc key to end the Shaft function |  |
| ### Modification : We are going to correct the mistake done on step 13; the actual diameter is 170, we typed a value of 150!. - Left-click twice directly on 150 or Right-click on the value then choose ‘Edit element’ - Replace the value of 150 with 170 in the field, and enter - Left-click on the Green Tick to validate |  |
| ### Finishing, definition of fillets for the groove: - Left-click on Zoom , then click on the screen twice to select the zone you want to zoom like shown on the picture - Click on Esc key to end the zoom function - Left-click on the Fillets function - To create a fillet you can either click directly on the corner where the fillet is or click on the 2 elements whose intersection is the fillet. - Type in 0.5 and press Enter. - Note : the decimal symbol is the point, not the comma. - Left-click on the 2 corners of the groove to apply the fillet. - Press ESC key to end the Fillets function - Left-click on - The wireframe design is finished. |  |

# Operating Procedure for the Machining:

Targets:

- Application of cycles for turning and grooving :

- Selection of geometry for machining

- Simple cycle parameters definition

- Selection of tool

- Use machining tree: modifying a cycle

- Tool path simulation

| ### Turning: - Left-click on Turning - Left-click on The stock is automatically created in relation to the profile. It is a cylindrical stock defined with a constant 5 mm overflow around the profile. We are going to modify the stock manually. - In the Zmaxi field, type 2 to make the stock more than 2mm behind the origin - The stock is cored out: - Change the value in D inner field to 70 and press ENTER - Left-click on to validate |  |
| --- | --- |

Ope 10 Facing

| - Selection of geometry: - Left-click on Note: It is recommended to always zoom during selection to facilitate selection of profiles. - Left-click on Selection - Left-click on the first element of the path as in the drawing and ensure that the arrow is pointing downwards. - Left-click on Profile OK - No need to modify the approach and return vectors which are tangent to the element shown. |  |
| --- | --- |
| - Tool selection: - Left-click on , then on Turning Facing Cutter - Left-click on - Select MT_Sandvik_2020_Right and - MT_Sandvik_2020_Left. - Left-click on Confirm to download the tools library. - After downloading and installing, select MT_Sandvik_2020_Left in the library - In the list of tool, Left-click on - " MS PCLNL 2020K 12" |  |
| - Selection of the machining cycle: - Left-click on then on Roughing - In the list of techno, Left-click on "Main Spindle - Face" |  |
| - Toolpath calculation : - Left-click on Cycle Calculation - Right-click on the cycle and rename it to ‘Facing’ |  |

Ope 20 Straight cutting (roughing)

| - Selection of geometry : - Left-click on - Left-click on Selection - Left-click on the first path element, on the right (image A), click on OK - Left-click on the last path element, on the left (image B), click on OK - Left-click on Profile OK - Note: No need to modify the approach vector which are tangent to the element shown. - Select the Ret Vect check box, then manually enter the value 90 and press Enter |  |
| --- | --- |
| - Selection of Tool : - Left-click on , then on Turning Facing Cutter - Select MT_Sandvik_2020_Left under user - In the list of tool, Left-click on " MS PDJNL 2020K 11" |  |
| - Selection of machining cycle : - Left-click on then on Roughing - In the list of techno, Left-click on "Main Spindle - Outside" |  |
| - Toolpath calculation : - Left-click on Cycle Calculation The toolpath is drawn, the evolutive stock is machined. - Right-click on the cycle and rename it to ‘Outside Roughing’ |  |

Ope 30 Straight cutting (finish)

| - Selection of geometry : The menu is still active. - Left-click on - In the machining tree left-click on to expand the details for the outside roughing cycle - Left-click and drag and drop the profile on the screen. Note: This method will copy the profile selection of the roughing cycle to the one currently being defined thus avoiding making the same selection multiple times. |  |
| --- | --- |
| - Selection of Tool : - Left-click on , then on Turning Facing Cutter - Select MT_Sandvik_2020_Left under user - In the list of tool, left click on " MS PDJNL 2020K 11" |  |
| - Selection of machining cycle : - Left-click on Cycles then on Finish - In the list of techno, left-click on "Main Spindle - Outside" |  |
| - Toolpath calculation : - Left-click on Cycle Calculation - Right-click on the cycle and rename it to ‘Outside Finish’ |  |

Ope 40 Groove cutting

| - Start: - Left-click on - Left-click on Zoom then click twice on the screen to select the area you want to zoom like shown on the picture. |  |
| --- | --- |
| - Selection of geometry : - Left-click on - Left-click on the first side of the groove (image A) - Left-click on the last side of the groove (image B) - Left-click on Profil OK - No need to modify the approach and return vectors which are tangent to the elements shown. |  |
| - Selection of Tool : - Left-click on Groove cutter - Select MT_Sandvik_2020_Left under user - In the list of tool, left mouse click on " MS LF123J13-2020BM" |  |
| - Selection of machining cycle : - Left-click on , then on Direct |  |
| - Toolpath calculation : - Left-click on Cycle Calculation |  |

Ope 50 Boring

| ### Selection of geometry : - Left-click on - Left-click on geometry selection - Left-click on the first element of the path, right (Figure A) - Left-click on the last element of the path, left (Figure B) - Left-click on Profil OK |  |
| --- | --- |
| ### Selection of Tool : - Left-click on tools , then on Boring cutter - Left-click on - Select ST_Sandvik_D12_Right and - ST_Sandvik_D12_Left. - Left-click on Confirm to download the tools library. - After downloading and installing, select ST_Sandvik_1212_Left - In the list of tool, select MS A12M-SCLCL 06-R |  |
| ### Selection of machining cycle : - Left-click on Cycles , then on Roughing - In the list of techno, left mouse click on "Main Spindle - Outside" |  |
| - Toolpath calculation : - Left-click on Cycle Calculation - Right-click on the cycle and rename it to ‘Inside Roughing’ |  |

Simulation and NC blocks output

| - Simulation : - Left-click on - Left-click on Simulation - Left-click on to activate the step by step mode. Then press the space key to go forward in the simulation step by step. - If you prefer continuous simulation, click on - Left-click on or Esc to stop the simulation - You can rotate the part by holding the left-click and moving the mouse. |  |
| --- | --- |
| - NC Code: - Left-click on - Among the proposed list, left-click on the post-processor "T67_Fanuc" - Left-click on Open - Left-click on Confirm The NC Code is generated. |  |

## C axis operations

| ### Switching to Reference Plane for Milling: - Go back to Design menu - Left-click on current plane tab which is in the Revolution plan and change it to Reference plane |  |
| --- | --- |
| ### Design of a hole: - Left-click on Wireframe, then select Holes - Left-click on the function Standard Holes - Select type of holes and choose Through smoothed hole - Left-click twice in the Diameter box and type 10 - Left-click twice in Altitude box and type -100 - Type the value for X as 292.5/2 and press Enter - Type the value for Y as 0 and press Enter - Left-click on |  |
| ### Start the milling: - Left-click on Milling - Left-click on Hole/Feature, then select Manual Holes |  |
| ### Selection of geometry : - Left-click on - Left-click on the hole |  |
| ### Selection of Tool : - Left-click on tools , then on drill - Left-click on - Select Standard - Left-click on Confirm to download - After the download and installation, select Standard under User - Left-click on "Drill D10" |  |
| ### Selection of machining cycle : - Left-click on Cycles then on Drilling - In the list of techno, Left-click on Simple mode Auto |  |
| - Toolpath calculation : - Left-click on Cycle Calculation - Right-click on the cycle and rename it to ‘Drilling D10’ The path is displayed on the screen. |  |
| ### Repeat the cycle in the machining tree: - Left-click on Drilling D10 cycle in machining tree - Select Operation Copy - Choose Rotation - Left-click twice in Number field, type in 11 and press Enter - Check that the coordinates of the center are 0, 0, 0 - Left-click twice in Angle field, type in 30, Enter key - Left-click on on Drilling in the machining tree to position the subprogram call after the drilling The 12 holes are now machined. |  |
| - Simulation : - Left-click on - Left-click on Simulation - Left-click on |  |
| - NC Code : - Left-click on - Among the proposed list, click on the post-processor T67_Fanuc - Left-click on Open - Left-click on Confirm The NC Code is generated. |  |