W02

# GO2cam V6.10

# Tutorial

# W02 – Taper Cut

## Process of Design:

| ### Start a new drawing: - Left click on File - Left click on New |  |
| --- | --- |
| ### Start: - Left click on Wireframe - Left click on Creation |  |
| ### Creation of cycle: - Left click on the function Cycle - Left click on GO2cam origin - Left click on the drawing interface. - Enter the radius value of 10 |  |
| ### Create 4 parallel lines: - Left click on the function Line - Left click on vertical aixs - Left click on the right side of the vertical axis - Type value 3 - Use same way to create two more vertical lines 15 mm and 0 mm to the right of vertical axis, and three horizontal lines 0 mm, 3 mm and 15 mm above horizontal axis |  |
| ### Limit the geometry: - Left click on then create a window to include the elements to be scaled - Left click on the function Drawing - Left click on items to be deleted, GO2cam displays the items you will keep in white color. Note: An opposite method is to hold the Shift key on the keyboard and click the item you want to keep. - Use the eraser icon to delete the lines on the axes. |  |
| ### Create a 3mm fillet: - Left click on Wireframe - Left click on Drawing Finish - Left click on Fillet - Type value 3 of radius - Left click on these 2 corners |  |
| ### Copy of rotation: - Right click in the background and select Duplicate Geometry. - Make a selection box to include the geometry. - Choose Rotation for the Mode. - Double left click in the box Angle and type 90 - Type 3 in the number of copy. - Then click on Validate . |  |
| ### Create threading points: - Left click on Wireframe - Left click on Creation - Left click on Points - Type value X = 5 and Y = 5 - Then click on confirm |  |
| ### Save file: - Left click on the menu File - Left click on “Save as”, and name it to “W02_Taper Cut” - Left click on Save. |  |

- Process of machining:

| ### Definition of threading point: - Return in menu Design - Enter in menu Threading - Left click on the function Manual Threadings and choose « Auto-Threadings on profiles » - Enter 2 mm for slug. - Choose mode "Die" and type value for distance 5 mm. - Left click on the profile, threading point is automatically created - Validate. |  |
| --- | --- |
| ### EDM: - Left click on icon The stock is automatically created based on your geometry. Note: This is a default definition of parallelepipedical stock, with a constant overflow of 5 mm around the geometry. - Modification of stock: - Left click twice in box Zmini and then type 0 - Left click twice in box Zmaxi and then type 30 - Left click on to validate the creation of stock. |  |
| ### Change views : Two possible ways to change views: - Left click on or right click on centre of the interactive axis, and then select the Isometric view |  |
| ### Applying a complete cycle: - Left click on - Left click on icon - Choose mode "Die" - Left click on the profile Note: Because of the machining contour, the thread point is automatically selected. - Enter the value 5 for both Top Z and Taper. Note: If the profile is clockwise, type value – 5 in box of Taper, or reverse the profile. - Verify the side of the taper |  |
| ### Selection of tool: - Left click on - In the list of tool, select tool "Cobra Cut 0.25" |  |
| ### Selection of cycle: - Left click on - Left click on Taper Cutting |  |
| ### Calculation of Cycle : - Left click on Cycle Calculation |  |
| ### Applying a straight cut cycle - Left click on - Left click on - Choose mode "Die" - Select the profile - Check whether the height is 0 |  |
| ### Selection of cycle: There is no longer a need to define a tool because it has been selected during the last cycle. - Left click on - Left click on Straight Cut - Left click on Cycle Calculation |  |
| ### Simulation : - Left click on - Left click on - Left click on to start simulation on all machining operations - Click the space bar or click on if you want to switch to step-by-step mode - Left click on or Press Escape to stop the simulation. |  |
| ### Generation of ISO program: - Left click on - From the suggested list, select the post-processor »E60_Charmilles_Robofil » - Open and confirm ISO program is generated. |  |

## Multi-pass machining procedure:

Save your file then re-open it. This time, keep the threading point and delete the machining.

| ### Application first pass "Complete + Stop": - In - Left click on - Left click on - Select the profile - Choose mode "Die" - Select cutting type "Complete+Stop" Note: The stop value of Slug has been defined when creating the threading point. - Enter value 5 in box Taper - Verify the side of the taper |  |
| --- | --- |
| ### Selection of tool: - Left click on - In the list of tool, select "Cobra Cut 0.25" |  |
| ### Selection of cycle: - Left click on - Left click on Taper Cutting - Left click on Cycle Calculation |  |
| ### Simulation : - Left click on - Switch the simulation to toolpath mode to view the stop - Left click on to start the simulation Note: In order to visualize the stop during the simulation, you must choose toolpath mode. Dynamic mode will not simulate the stop. |  |
| ### Applying a second « complete cut profile » : - Left click on - In the machining tree, drag profile icon into the middle of the screen - Left click on - Change the cutting type to « Cut of the complete profile » - Verify whether other settings remain unchanged - Left click on Cycle Calculation |  |

| ### Applying a Third toolpath of "Cut of complete profile in inverse direction" : - Left click on - In the machining tree, drag profile icon into the middle of the screen - Left click on the icon - Change the cutting type to "Cut of the complete profile in inverse direction" - Check whether other settings remain unchanged - Select the strategy - Left click on Cycle Calculation |  |
| --- | --- |
| ### Simulation : - Left click on - Choose mode "Dynamic" or "Toolpath" - Left click on to start simulation |  |