W06

GO2cam V6.08

Tutorial solid

W06 – Taper Cut

## Process of Solid import:

| ### Repositioning: - In the menu File, Left click on - Open file: W06 - Solid Taper Cut.X_T - Left click on the support face with the icon - Validate ### Definition of Stock: Automatically create stock regarding your solid. Note: It is a parallelepipedical stock defined by default, and has a constant overflow of 5 mm around the solid. - Left click on in order to make the height of stock equal to the height of solid, the Height turns grey - Change the overflow value to 0 - Left click on to validate ### Positioning of origin: GO2cam saves at the last location used for the origin. If your origin position is still in the same location, you can directly verify this step. - Left click on , choose center of part - Left click on to finish the solid import Solid is imported and repositioned, stock is defined relative to the part, and the origin is positioned. |  |
| --- | --- |

## Create machining profile on topology:

| ### 1. Definition of Profile: - In Design, enter in menu EDM Geom - Left click on Straight/Taper on solid - Left click on the start face of Profile - Left click on the arrow to define the direction of the profile. - Left click on the same face to close the profile - A selection appears and the starting position of the profile is displayed in green, Left click on to validate The profile is displayed in greenish-yellow, and the green flag indicates the start and end positions. ### Create a 4-axis profile: We will process the taper in 4-axis mode. - Left click on 4 axis on solid - Choose mode Chaining of faces - Left click on above taper surface - Check direction of the arrow is the same as the previous profile - Left click on the same face to close the profile The top and bottom profiles appear, and we can view the white automatic markers. - Left click on ### Save of file: - Left click on the menu File - Left click on Save as - Type name : W06 - Solid Taper Cut - Left click on Save |  |
| --- | --- |
| ### Definition of threading point: - Return in the menu Design - Enter in menu Threading - Left click on the function Manual Threadings and choose « Auto-Threadings on profiles » - Type value 3 for Slug, - Choose mode Die - Type value 5 for the Distance. - Left Click the profile to automatically create the threading point - Validate Your part is ready for machining. |  |

## Machining Procedure:

| ### Application of a Complete Cutting Cycle: - Left click on - Left click on the icon - Left click on the bottom profile Note: Because of the machining profile, the thread point is automatically selected. | Maybe to re-consider this image |
| --- | --- |
| ### Selection of tool: - Left click on - In the list of tool, select tool "Cobra Cut 0.25" |  |
| ### Selection of cycle: - Left click on - Left click on Straight Cut |  |
| ### Calculation of Cycle : - Left click on Cycle Calculation |  |
| ### Toolpath in 4 Axis: - Left click on - Left click on - Select the top or bottom profile of the taper surface The height, angle, and threading point are automatically created. |  |
| ### Selection of cycle: It is no longer necessary to define a tool because it has been selected in last operation. - Left click on - Left click on 4 axes cnt/cnt |  |
| ### Calculation of cycle: - Left click on Cycle Calculation |  |

| ### Simulation : - Left click on - Left click on - Left click on to start simulation on all machining operations - Click the space bar or click on if you want to switch to step-by-step mode - Left click on or Press Escap to stop the simulation. |  |
| --- | --- |
| ### Generation of ISO program: - Left click on - From the suggested list, go into the lib folder then select the post-processor «EROBOF44«. - Open and confirm ISO program is generated. |  |

## Multi-pass machining procedure:

Save the PCE file and re-open it without the toolpath.

| ### 1. Applying first pass "Complete + Stop": - In - Left click on - Left click on - Select the top or bottom profile of the taper surface - Select cutting type "Complete+Stop" Note: The stop value corresponds to the slug value entered when creating the threading point. |  |
| --- | --- |
| ### 2. Selection of tool: - Left click on - In the list of tool, select "Cobra Cut 0.25" |  |
| ### 3. Selection of cycle: - Left click on - Left click on 4 axes cnt/cnt - Left click on Cycle Calculation |  |

| ### 4. Simulation : - Left click on - Switch the simulation to toolpath mode to view the stop - Left click on to start the simulation Note: In order to visualize the stop during the simulation, you must choose toolpath mode. Dynamic mode will not simulate the stop. |  |
| --- | --- |
| ### Second pass of "Cut of the complete profile": - Left click on - Left click on - In the machining tree, drag profile icon into the middle of the screen - Change the cutting type to « Cut of the complete profile » - Check whether other settings remain unchanged - Select strategy 4 axes cnt/cnt, the tool has been defined at the first cycle - Left click on Cycle Calculation |  |

| ### Third pass of "Cut of complete profile in inverse direction" : - Left click on - Left click on the icon - In the machining tree, drag profile icon into the middle of the screen - Change the cutting type to "cut of the complete profile in inverse direction" - Check whether other settings remain unchanged - Select the strategy 4 axes cnt/cnt - Left click on Cycle Calculation |  |
| --- | --- |
| ### Simulation : - Left click on - Choose mode "Dynamic" or "Toolpath" - Left click on to start simulation |  |