W04

GO2cam V6.10

Tutorial

W04 – 4 Axis

# Design:

| ## Start a new drawing: - Left click on File, then choose New - Save current file |  |
| --- | --- |
| ## Sketch: - Left click on Wireframe - Left click on Creation |  |
| ## Creation of bottom profile, Spade: The reference plane is active by default. We will create the first shape in it. - Use the drawing command to create the shape of Spade. - Kindly refer to the 4-Axis Layout Document provided. |  |

| ## Limitation of lines: - Left click on - Click the item you want to delete, and GO2cam will highlight the item you want to keep - You have another option, hold down the Shift key and click at the item you want to keep - Left click on Framing to have all geometry on the frame screen |  |
| --- | --- |
| ## Creation of taper: - In the menu Wireframe select Drawing Finish - Left click on Fillet - Type 0.5 of Fillet - Left click on the 2 intersections |  |

| ## Creation of threading points : - In Wireframe, Choose Creation - Left click on Point - Type 0 in X, - Type -23 in X, - Click on xyz validate - Enter menu Threading - Left click on the command Manual Threadings and choose « Manual threading » - Settings of threading points : - Value of Slug = 2 - Value of Retract = 0.5 - Left click on the created point Note: For 4 axis, threading points must be created manually. |  |
| --- | --- |
| ## Divide geometry: - Return to Drawing Finish - Left click on Divide - Left click on Elements to be divided - Left click on Element, point or axis that the segment must be divided. - Right-Click and Validate. |  |
| ## Creation of top profile, clover: - Left click on list of views, then select the TOP REFERENCE EDM - Create a clover shape using the drawing command. - Kindly refer to the 4-Axis Layout Document provided. |  |
| ## Finishing: - Use command Drawing to obtain the final shape of clover. - Create 1 mm Fillet - Divide the geometry relative to threading point. |  |
| ## Change altitude: - Right click on list of views, select Edit current plane - Enter the profile Z value in Z 30 - Left click on Ok to validate |  |

# Single-pass 4-axis machining:

| ### Application of machining : ## Start: - In the menu Automatically create stock based on your geometry. Note: This is a default definition of parallelepiped stock, with a constant overflow of 5 mm around the geometry - Left click on to make the height of stock equal to the height of two geometric shapes Validate. |  |
| --- | --- |
| ## Machining : - Left click on the menu - Left click on selection of profile - First select the clover at the top (top profile) - Left click on , then on and validate - Select the spade at the bottom for the second profile (bottom profile) - Left click on , then on to validate - Choose mode Punch - Check whether the Top Z and Bottom Z are correct - Left click on Threading Point , then click on the point |  |
| ## Selection of Tool: - Left click on - In the tool list, choose the tool Cobra Cut 0.25 |  |
| ## Selection of cycle : - Left click on - Select cycle 4 axes Cnt/Cnt |  |
| ## Calculation of cycle: - Left click on Cycle Calculation We can see that the wire is delayed on the bottom profile, so we will place a marker to synchronize the wire's passing on the top and bottom profile. |  |
| ### Synchro Points: ## Start: - In the menu design - Left click on |  |
| ## Add Synchro Points: - Use command Synchro points - Left click on Top profile, then bottom profile - Check whether the mark is in punch mode - Click to allow automatic linking of all existing points - The command adding synchros is activated, just click one end of the top profile and connect it to the end of the bottom profile - If the end does not exist, use command cut profile so that synchro point can be created at the ends - Left click on to validate |  |
| ## Update toolpath : - Return in menu - Left click on update , then on Total Toolpath is recalculated based on added synchro points. |  |

| ### Simulation and NC output: - Simulation : - Enter in menu - Left click on Simulation - Left click on Or click the space bar to start the simulation in step-by-step mode - Left click on Or press the Escape key to stop the simulation. |  |
| --- | --- |
| ## Generation of NC program : - Left click on - In the proposed list, Left click on the post-processor »E60_Charmilles_Robofil ». - Open and confirm ISO program is generated. - Left click on Open, then on Confirm The ISO program is generated. |  |